Supplementary Material

Ozone Injection System Based on NETmix Technology for Quaternary Treatment of Urban Wastewater

Mateus Mestriner Pitucoa,b, Paulo H. Marrocosa,b, Sandra Méndezc, Rosa Montesc, Rosario Rodilc, Francisca C. Moreiraa,b, Vítor J.P. Vilara,b*

a LSRE-LCM – Laboratory of Separation and Reaction Engineering - Laboratory of Catalysis and Materials, Faculty of Engineering, University of Porto, Rua Dr. Roberto Frias, 4200-465, Porto, Portugal

b ALiCE – Associate Laboratory in Chemical Engineering, Faculty of Engineering, University of Porto, Rua Dr. Roberto Frias, 4200-465, Porto, Portugal

c Department of Analytical Chemistry, Nutrition and Food Sciences, Institute of Research on Chemical and Biological Analysis (IAQBUS), Universidade de Santiago de Compostela, Constantino Candeira S/N, 15782 Santiago de Compostela, Spain

*Corresponding author:

E-mail address: vilar@fe.up.pt

Table S1. Analytical determinations.

| Parameter (units) | Analytical method |
| --- | --- |
| Conductivity (µS cm-1) | Conductivity was determined by Hanna Instruments HI9828 multiparameter. |
| pH and temperature (oC) | pH and temperature were measured by a Hanna Instruments HI-2020 edge® hybrid. |
| Turbidity | Turbidity was measured by a Hanna Instruments HI 88703 turbidimeter. |
| Total dissolved carbon (g m-3)* | Total dissolved carbon and dissolved inorganic carbon were determined by NDIR spectrometry in a TC-TOC analyzer equipped with ASI-V autosampler (Shimadzu, model TOC-VCSN). Dissolved organic carbon was given by the difference between these two parameters. |
| Dissolved organic carbon (g m-3)* |  |
| Dissolved inorganic carbon (g m-3)* |  |
| Chemical oxygen demand (g m-3) | Chemical oxygen demand (COD) was determined by the dichromate closed reflux method according to the SMEWW - 5220 D test [1], digested in a WTW CR4200 thermoreactor, followed by photometric measuring (Merck Spectroquant® Prove 300 UV/Vis spectrophotometer). |
| Absorbance at 254 nm (UVA)* | The ultraviolet absorbance (UVA) was determined with a Merck Spectroquant® Prove 300 UV/Vis spectrophotometer. |
| Total suspended solids (g m-3) | Total suspended solids (TSS) were determined according to the SMEWW 2540 D test [1], by filtration through a weighted standard glass-fiber filter and drying of the solid residue at 105 °C up to constant weight. |
| Sodium, Na+ (g m-3)* | Dissolved inorganic anions were analyzed by ion chromatography (Dionex ICS-2100 LC) equipped with an IonPac® AS11-HC 250 mm × 4 mm column (T = 30 ºC) and a self-regenerating anion suppressor (ASRS® 300, 4 mm) under an isocratic elution of 30 mM KOH at a flow rate of 1.5 mL min-1. Dissolved inorganic cations were also evaluated by ion chromatography in a Dionex DX-120 LC coupled with an IonPac® CS12A 250 mm × 4 mm column at ambient temperature and a CSRS® Ultra II cation self-regenerating suppressor (4 mm) under an isocratic elution of 20 mM methanesulfonic acid at a flow rate of 1.0 mL min-1. |
| Ammonium, NH4+ (g m-3)* |  |
| Potassium, K+ (g m-3)* |  |
| Magnesium, Mg2+ (g m-3)* |  |
| Calcium, Ca2+ (g m-3)* |  |
| Chloride, Cl− (g m-3)* |  |
| Sulfate, SO42− (g m-3)* |  |
| Nitrite, NO2- (g m-3)* |  |
| Nitrate, NO3- (g m-3)* |  |
| Phosphate, PO43− (g m-3)* |  |

* Samples were filtrated using 0.45 µm Nylon filters from VWR before analysis.

Table S2. Limit of quantification (LOQ) for 19 target CECs spiked to SW, UWW1, and UWW2 water matrices according to LC-MS/MS method performance.

| Contaminant |  | SW | UWW1 | UWW2 |
| --- | --- | --- | --- | --- |
|  |  | LOQ | LOQ | LOQ |
|  |  | µg dm-3 | µg dm-3 | µg dm-3 |
| Carbamazepine | CBZ | 0.02 | 0.03 | 0.1 |
| 17β-Estradiol | E2 | 2.10 | 3.10 | 2.5 |
| 17α-Ethynylestradiol | EE2 | 1.50 | 2.30 | 2.0 |
| Diclofenac | DCF | 0.18 | 0.19 | 1.0 |
| Losartan | LSTN | 0.15 | 0.26 | 0.1 |
| Bisoprolol | BSPL | 0.04 | 0.11 | 0.05 |
| Atenolol | ATNL | 0.15 | 0.24 | 0.1 |
| Acesulfame | AC-K | 0.50 | 0.68 | 3.0 |
| Valsartan | VSTN | 0.28 | 0.44 | 0.1 |
| Irbesartan | ISTN | 0.05 | 0.08 | 0.1 |
| Diuron | DRN | 0.07 | 0.07 | 0.1 |
| Diethyltoluamide | DEET | 0.03 | 0.03 | 0.0 |
| Carbamazepine 10.11-epoxide | CBZ-E | 0.02 | 0.02 | 0.2 |
| Saccharin | SAC | 1.00 | 1.10 | 0.1 |
| Melamine | MAL | 0.58 | 0.75 | 0.5 |
| Pentadecafluorooctanoic acid | PFOA | 0.02 | 0.02 | 0.1 |
| Perfluorobutane sulfonate | PFBS | 0.09 | 0.09 | 0.2 |
| Heptafluorobutyric acid | PFBA | 0.14 | 0.14 | 0.10 |
| Trifluoromethanesulfonic acid | TFMS | 1.00 | 1.20 | 0.5 |

Table S3. Concentration of CECs analyzed in raw wastewater, treated wastewater (WWTPII effluent), MBR inlet, and MBR outlet (post-MBR treatment – UWW2) based on screening campaign; and respective removal rate obtained.

| Contaminant |  | LOQ | Raw WW | Treated WW | WWTPII removal | MBR Inlet | MBR Outletα | MBR removal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | µg dm-3 | µg dm-3 | µg dm-3 | % | µg dm-3 | µg dm-3 | % |
| Pharmaceuticals |  |  |  |  |  |  |  |  |
| Amantadine* | AMT | 0.0003 | 0.53 | 0.34 | 36% | 0.32 | 0.33 | 0% |
| Amisulpride* | ASPD | 0.0025 | 0.55 | 0.39 | 28% | 0.42 | 0.40 | 4% |
| Atenolol* | ATNL | 0.0068 | 0.06 | 0.04 | 20% | n.d | 0.04 | - |
| Bisoprolol | BSPL | 0.05 | n.a | n.a | - | n.a | 0.33 | - |
| Carbamazepine* | CBZ | 0.0005 | 0.27 | 0.21 | 22% | 0.19 | 0.38 | 0% |
| Diclofenac | DCF | 1.00 | n.a | n.a | - | n.a | 2.35 | - |
| Flecainide* | FLE | 0.0041 | 0.18 | 0.14 | 18% | 0.18 | 0.13 | 28% |
| Irbesartan* | ISTN | 0.0025 | 6.61 | 4.18 | 37% | 8.08 | 3.57 | 56% |
| Lamotrigine* | LAM | 0.0126 | 0.64 | 0.55 | 13% | 0.55 | 0.61 | 0% |
| Losartan | LSTN | 0.10 | n.a | n.a | - | n.a | n.d | - |
| Metformin* | MET | 0.08 | 66.8 | 6.16 | 91% | 1.70 | 1.85 | 0% |
| Sulpiride* | SPD | 0.0009 | 0.23 | 0.07 | 68% | 0.10 | 0.07 | 27% |
| Tramadol* | TRA | 0.0023 | 0.56 | 0.59 | 0% | 0.61 | 0.55 | 10% |
| Tiapride* | TPD | 0.0007 | 0.83 | 0.74 | 10% | 0.96 | 0.66 | 32% |
| Valsartan | VSTN | 0.10 | n.a | n.a | - | n.a | n.d | - |
| Venlafaxine* | VEN | 0.0027 | 1.13 | 0.94 | 17% | 1.04 | 0.71 | 31% |
| Endocrine disrupting compounds |  |  |  |  |  |  |  |  |
| 17α-Ethynylestradiol | EE2 | 2.00 | n.a | n.a | - | n.a | n.d | - |
| 17β-Estradiol | E2 | 2.50 | n.a | n.a | - | n.a | n.d | - |
| Pharmaceutical metabolites |  |  |  |  |  |  |  |  |
| Carbamazepine 10,11-epoxide | CBZ-E | 0.20 | n.a | n.a | - | n.a | 0.24 | - |
| O-desmethylvenlafaxine* | ODV | 0.0075 | 3.25 | 3.28 | 0% | 3.18 | 2.53 | 20% |
| Biocides — pesticides |  |  |  |  |  |  |  |  |
| Diethyltoluamide* | DEET | 0.0037 | 0.52 | 0.42 | 19% | 0.23 | 0.42 | 0% |
| Diuron* | DRN | 0.0018 | 0.02 | 0.03 | 0% | 0.04 | 0.02 | 40% |
| Terbutryn* | TBT | 0.0023 | 0.12 | 0.05 | 61% | 0.06 | 0.04 | 27% |
| Food additives |  |  |  |  |  |  |  |  |
| Acesulfame K* | AC-K | 0.0017 | 58.3 | 0.27 | >99% | 0.04 | 2.25 | 0% |
| Caffeine* | CAFF | 0.0003 | 19.0 | 0.07 | >99% | 0.11 | 0.09 | 20% |
| Saccharin | SAC | 0.10 | n.a | n.a | - | n.a | 8.40 | - |
| Other common chemicals Industrial chemical |  |  |  |  |  |  |  |  |
| Melamine | MEL | 0.50 | n.a | n.a | - | n.a | 5.77 | - |
| Tris (2-chloroisopropyl) phosphate* | TCPP | 0.0340 | 1.60 | 0.46 | 71% | 0.37 | 0.86 | 0% |
| Short-chain perfluoroalkyl substances (PFAS) |  |  |  |  |  |  |  |  |
| Heptafluorobutyric acid | PFBA | 0.10 | n.a | n.a | - | n.a | 0.16 | - |
| Pentadecafluorooctanoic acid | PFOA | 0.10 | n.a | n.a | - | n.a | n.d | - |
| Perfluorobutane sulfonate | PFBS | 0.20 | n.a | n.a | - | n.a | n.d | - |
| Trifluoromethanesulfonic acid | TFMS | 0.50 | n.a | n.a | - | n.a | n.d | - |

* CECs measured according to the method proposed in Montes et al. [2]. LOQ – Limit of quantification; α Refers to UWW2 (post-MBR effluent). n.a – not analyzed; n.d – not detected.

Table S4.  Target CECs for contamination/quantification in this work: acronym, molecular weight (MW), CAS number, and reactivity of the moieties with O3 (at circumneutral pH).

| Contaminant |  | CAS number | MW (g mol-1) | Reactive moieties | Reactivity with O3 |
| --- | --- | --- | --- | --- | --- |
| Pharmaceuticals |  |  |  |  |  |
| Amantadine | AMT | 768-94-5 | 151.2 | Primary amine | Resistant |
| Amisulpride | ASPD | 71675-85-9 | 369.5 | Tertiary amine | Reactive |
| Atenolol† | ATNL | 29122-68-7 | 266.3 | Secondary amine | Moderate |
| Bisoprolol† | BSPL | 66722-44-9 | 325.4 | Aromatic ring | Reactive |
| Carbamazepine† | CBZ | 298-46-4 | 236.3 | Olefin, amide, benzene ring | Reactive |
| Diclofenac† | DCF | 15307-86-5 | 296.2 | Aniline | Reactive |
| Flecainide | FLE | 54143-55-4 | 414.1 | Secondary amine | Reactive |
| Irbesartan† | ISTN | 138402-11-6 | 428.5 | None | Resistant |
| Lamotrigine | LAM | 84057-84-1 | 255.0 | None | Resistant |
| Losartan† | LSTN | 124750-99-8 | 422.9 | Imidazole, benzene ring | Reactive |
| Metformin | MET | 657-24-9 | 129.2 | None | Resistant |
| Sulpiride | SPD | 15676-16-1 | 341.1 | Tertiary amine | Reactive |
| Tramadol | TRA | 27203-92-5 | 263.4 | Tertiary amine | Moderate |
| Tiapride | TPD | 51012-32-9 | 328.4 | None | Resistant |
| Valsartan† | VSTN | 137862-53-4 | 435.5 | None | Resistant |
| Venlafaxine | VEN | 93413-69-5 | 277.4 | Tertiary amine | Moderate |
| Endocrine disrupting compounds |  |  |  |  |  |
| 17α-Ethynylestradiol† | EE2 | 57-63-6 | 296.4 | Phenol | Reactive |
| 17β-Estradiol† | E2 | 50-28-2 | 272.4 | Phenol | Reactive |
| Pharmaceutical metabolites |  |  |  |  |  |
| Carbamazepine 10,11-epoxide† | CBZ-E | 36507-30-9 | 252.3 | None | Resistant |
| O-desmethylvenlafaxine | ODV | 93413-62-8 | 263.4 | Tertiary amine, phenol | Reactive |
| Biocides — pesticides |  |  |  |  |  |
| Diethyltoluamide† | DEET | 134-62-3 | 191.3 | None | Resistant |
| Diuron† | DRN | 330-54-1 | 233.1 | Benzene ring | Resistant |
| Terbutryn | TBT | 886-50-0 | 241.4 | None | Resistant |
| Food additives |  |  |  |  |  |
| Acesulfame K† | AC-K | 33665-90-6 | 163.1 | None | Resistant |
| Caffeine | CAFF | 58-08-2 | 194.2 | Olefine | Moderate |
| Saccharin† | SAC | 81-07-2 | 183.2 | None | Resistant |
| Other common chemicals Industrial chemical |  |  |  |  |  |
| Melamine† | MEL | 108-78-1 | 126.1 | None | Resistant |
| Tris (2-chloroisopropyl) phosphate | TCPP | 13674-84-5 | 327.6 | None | Resistant |
| Short-chain perfluoroalkyl substances (PFAS) |  |  |  |  |  |
| Heptafluorobutyric acid† | PFBA | 375-22-4 | 214.0 | None | Resistant |
| Pentadecafluorooctanoic acid† | PFOA | 335-67-1 | 414.1 | None | Resistant |
| Perfluorobutane sulfonate† | PFBS | 45187-15-3 | 299.1 | None | Resistant |
| Trifluoromethanesulfonic acid† | TFMS | 1493-13-6 | 150.1 | None | Resistant |

† CECs selected for contamination of water matrices (SW, UWW1, and UWW2).

Chapter S1. Determination of second-order rate constant for the reaction of SAC with O3

The apparent second-order rate constants at pH 2 and 7 for SAC were determined under pseudo-first-order conditions with an excess of ozone and by measuring the abatement of the target compound according to the absolute method [3]. The experiments were performed in ultrapure water using tert-butanol (t-BuOH, 5×10-3 M) as the ●OH radical scavenger. The solutions were adjusted to the desired pH (2 and 7) with phosphate buffer (2×10-2 and 5×10-1 M, respectively). The O3 reactions were carried out in a 1.0 dm3 borosilicate glass reactor, operated in semi-batch mode. O3 was continuously bubbled into ultrapure water (UPW) through a diffuser placed at the bottom of the reactor. The inlet O3 concentration and gas flowrate were kept constant at 20 g Nm-3 and 15 Ndm3 h-1, respectively. Once O3 saturation in the water was reached (O3* = ~1.0×10-4 M), SAC (C0 = 2.7×10-6 M) was added. The solution inside the reactor was maintained homogeneously stirred by a magnetic stir bar. Samples were withdrawn at regular time (t) intervals, and the dissolved O3 concentration ([O3]) and the residual concentrations of SAC were monitored.

SAC concentration was quantified with an Agilent 1100 series HPLC and performed on a reverse phase C18 analytical column (Phenomenex 3 × 150 mm, 5 µm). The mobile phase was a mixture of 98% of UPW (with phosphate buffer pH 7; 0.05%) and 2% of acetonitrile, eluted isocratically at a flowrate of 1 mL min−1. The detection wavelength was set at 220 nm and the injection volume at 100 μL. The LOQ of SAC was 6.5×10-8 M with this method. [O3] was determined by the indigo method at 600 nm [4].

The data were evaluated based on Eq. (S1), where kO3-SAC is the rate constants for SAC.

| – lnCC0= kO3-SACt0t[O3] dt | (S1) |
| --- | --- |

Considering that O3 dissolved in the solution was applied in excess compared to SAC (about 33-fold excess), a plot of the logarithm of the relative SAC concentration –lnCC0 versus the O3 exposure gave a linear function, of which the slope corresponds to the kO3-SAC at the given pH (Figure S1). As such, the measured apparent second-order rate constants at pH 2 and 7 were (2.4±0.1)×10-2 M-1 s-1 and (4.0±0.1)×10-1 M-1 s-1, respectively.

Figure S1. Plot of the logarithm of the SAC concentration (–ln (C/C0)) as a function of the O3 exposure (∫[O3]dt) for (a) pH = 2.0 and (b) pH = 7.0.

Chapter S2. Immediate ozone demand (IOD)

In wastewater effluents, many compounds, organic or inorganic, dissolved or suspended, may react with ozone with kinetic rate constants that span over a large range of values. As such, two parameters represent the ozone demand: (i) the immediate ozone demand (IOD), corresponding to the amount of ozone to apply before detecting ozone dissolved in the effluent. It reflects very fast reactions of the organic compounds (e.g., DOM) with ozone; and (ii) the decay kinetic constant (kd) due to overall ozone consumption by moderate or slow reactions, which is, in this case, a first-order rate constant [5,6]. According to Eq. (S2), a plot of [O3]L versus TOD yields a straight line with the horizontal axis, resulting in the value of IOD, whereas the slope allows calculating kd [7]:

| [O3]L = TOD1 + kd τ – IOD1 + kd τ | (S2) |
| --- | --- |

in which τ is the system residence time (55 s). Figure S2a shows the evolution of the [O3]L as a function of the TOD during ozonation of UWW2 (without spiked CECs). The IOD corresponds to a TOD value of 8.4 g m-3 and kd of  (9±5)×10-3 s-1. Similar ozone demand values were found in other studies of ozonation of UWW matrices [5,8]. Additionally, Figure S2b illustrates changes in the ozone transfer efficiencies during the ozonation tests. It is possible to observe that a high ozone transfer yield occurs until the IOD. Afterward, a minimal stabilization stage was expected, corresponding to a low ozone transfer yield and the simultaneous increase of the dissolved ozone concentration.

Figure S2. (a) Ozone residual concentration ([O3]L) versus transferred ozone dose (TOD); and (b) TOD and ozone transfer yield (ƞ) as a function of the applied ozone dose (AOD) during the ozonation of wastewater collected after MBR treatment (UWW2 without spiked CECs).

# References

[1]	American Public Health Association, Standard methods for the examination of water and wastewater, 23rd ed., American Public Health Association, Washington, D.C, 2017.

[2]	R. Montes, S. Méndez, J. Cobas, N. Carro, T. Neuparth, N. Alves, M.M. Santos, J.B. Quintana, R. Rodil, Occurrence of persistent and mobile chemicals and other contaminants of emerging concern in Spanish and Portuguese wastewater treatment plants, transnational river basins and coastal water, Sci. Total Environ. 885 (2023) 163737. https://doi.org/10.1016/j.scitotenv.2023.163737.

[3]	F.J. Beltran, Ozone reaction kinetics for water and wastewater systems, first ed., CRC Press, London, 2004.

[4]	H. Bader, J. Hoigné, Determination of ozone in water by the indigo method, Water Res. 15 (1981) 449–456. https://doi.org/10.1016/0043-1354(81)90054-3.

[5]	M. Marce, B. Domenjoud, S. Esplugas, S. Baig, Ozonation treatment of urban primary and biotreated wastewaters : Impacts and modeling, Chem. Eng. J. 283 (2016) 768–777. https://doi.org/10.1016/j.cej.2015.07.073.

[6]	A. Schmitt, J. Mendret, M. Roustan, S. Brosillon, Ozonation using hollow fiber contactor technology and its perspectives for micropollutants removal in water: A review, Sci. Total Environ. 729 (2020) 138664. https://doi.org/10.1016/j.scitotenv.2020.138664.

[7]	M. Roustan, H. Debellefontaine, Z. Do-Quang, J.-P. Duguet, Development of a method for the determination of ozone demand of a water, Ozone: Sci. Eng. 20 (1998) 513–520. https://doi.org/10.1080/01919519809480338.

[8]	A. Cruz-Alcalde, S. Esplugas, C. Sans, Continuous versus single H2O2 addition in peroxone process: Performance improvement and modelling in wastewater effluents, J. Hazard. Mater. 387 (2020) 121993. https://doi.org/10.1016/j.jhazmat.2019.121993.